02 485/TTL Communication Control Protocol V1.0

# 2.1 485 Communication Control Protocol V1.0

# 1 Introduction

This document defines the  MD series servo  communication control protocol format, with the protocol version being V1.0. This protocol is only applicable to driving MD series servos.

# 2 Data Frame Format

Communication transmission is asynchronous and in units of ByteDance. Each data frame of communication is a serial data stream of indeterminate length. The total length of the data frame is: (N + 2) ByteDance.

## 2.1 Basic Format

### 2.1.1 Instruction Package

Instruction Package Format:

| Format | Header | ID | Data Length | Instruction | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- | --- |
| Length (Byte) | 2 | 1 | N+2 | 1 | N | 1 |

The meaning of each ByteDance is as follows:

Header: 2 Byte, with fixed values of 0xFF, 0xFF.

ID : 1 ByteDance, this is the ID number of each joint servo, ranging from 0X00 to 0XFD. ID number 254 is the broadcast ID. If the ID number sent by the controller is 254 (0XFE), all servos will receive the command, and no response information will be returned for other commands except the PING command (the broadcast PING command cannot be used when multiple servos are connected to the bus);

Data Length: 1 ByteDance, equal to the parameter N to be sent plus 2, i.e., "N+2".

Instruction: Packet operation function code, see the following instruction types for details;

Parameter: N ByteDance, additional control information required besides the instruction. The parameter supports up to a double ByteDance parameter to represent a memory value, and the ByteDance order refers to the memory control table in the servo motor user manual (the ByteDance order varies for different servo motor models).

Checksum: 1 ByteDance, the calculation method of the checksum is as follows:

Check Sum = ~ (ID + Data Length + Instruction + Parameter), if the sum of the calculation within the parentheses exceeds 255, take the lowest ByteDance, and "~" indicates bitwise negation.

### 2.1.2 Response Packet

Response Packet Format:

| Format | Header | ID | Data Length | Status | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- | --- |
| Length (Byte) | 2 | 1 | N+2 | 1 | N | 1 |

The meaning of each ByteDance is as follows:

Header: 2 ByteDance, with fixed values of 0xFF, 0xFF;

Message Type: 1 ByteDance, which is the ID number of the returned joint servo;

Data Length: 1 ByteDance, equal to the parameter N to be sent plus 2, i.e., "N+2".

Status: The returned response packet contains the current state ERROR of the servo. If the current operating state of the servo is abnormal, it will be reflected through this ByteDance (for details on the meaning represented by each state, please refer to the Memory Control Table in the manual). If ERROR is 0, the servo has no error information. If the command is a read command (READ DATA), then Parameter1... Parameter N are the read information.

Parameter: N ByteDance, additional control information required besides the instruction. The parameter supports up to a double ByteDance parameter to represent a memory value, and the ByteDance order refers to the memory control table in the servo motor user manual (the ByteDance order varies for different servo motor models).

Checksum: 1 ByteDance, the calculation method of the checksum is as follows:

Check Sum = ~ (ID + Data Length + Instruction + Parameter),  if the sum of the calculation within the parentheses exceeds 255, then take the lowest ByteDance, and "~" indicates bitwise negation .

## 2.2 Instruction Type

Different instructions represent different operations, as detailed in the following table:

| Instruction | Function | Value | Parameter Length |
| --- | --- | --- | --- |
| PING (Query) | Query the working status | 0x01 | 0 |
| READ DATA | Query the characters in the control table | 0x02 | 2 |
| WRITE DATA | Write characters into the control table | 0x03 | Greater than or equal to 1 |
| REGWRITE DATA (Asynchronous Write) | Similar to WRITE DATA, but the control character does not act immediately after being written, until the ACTION instruction arrives | 0x04 | Not less than 2 |
| ACTION (Perform asynchronous write) | Trigger the action of REG WRITE | 0x05 | 0 |
| SYCNREAD DATA (Synchronous Read) | Used to query multiple servos simultaneously | 0x82 | Greater than or equal to 3 |
| SYCNWRITE DATA (Synchronous Write) | Used to control multiple servos simultaneously | 0x83 | Not less than 2 |
| RESET (Reset) | Reset the control table to factory defaults | 0x06 | 0 |

### 2.2.1 Query Status Command PING

Function Read the operating status of the servo motor

Length 0X02

Instruction 0X01

Parameter None

The PING command uses the broadcast address, and the servo also returns a response message.

Example: Read the operating status of the servo with ID number 1.

Command Frame:FF FF 01 02 01 FB (Sent in Hexadecimal)

| Header | ID | Data Length | Instruction | Checksum |
| --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X02 | 0X01 | 0XFB |

Returned Data Frame:  FF FF 01 02 00 FC  (Hexadecimal Display)

| Header | ID | Data Length | Status | Checksum |
| --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X02 | 0X00 | 0XFC |

### 2.2.2 Read Instruction READ DATA

Function   Read data from the servo memory control table

Length 0X04

Instruction 0X02

Parameter 1 The starting address of the data readout segment

Parameter 2 Length of Read Data

Example:  Read the current position of the servo with ID 1 (low byte first, high byte last).

Read two ByteDance from address0X38 in the control table.

Command Frame:FF FF 01 04 02 38 02 BE (Sent in Hexadecimal)

| Header | ID | Data Length | Instruction | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X04 | 0X02 | 0X38 0X02 | 0XBE |

Returned Data Frame:FF FF 01 04 00 18 05 DD (Hexadecimal Display)

| Header | ID | Data Length | Status | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X04 | 0X00 | 0X18 0X05 | 0XDD |

The two bytes of data read are: low byte L 0X18, high byte H 0X05. The two bytes combine to form a 16-bit data 0X0518, and the current position expressed in decimal is 1304.

### 2.2.3 Write Instruction WRITE DATA

Function Write data to the servo memory control table

Length N+3 (N is the parameter length)

Instruction 0X03

Parameter 1  The first address of the data write segment

Parameter 2  The first data written

Parameter 3  Second data

Parameter N+1 The Nth data

Example: Set an ID with an arbitrary number to 1.

The address where the ID number is stored in the control table is 5, so simply write 1 at address 5. The ID of the sent command packet uses the broadcast ID (0xFE).

Command Frame: FF FF FE 04 03 05 01 F4 (Sent in Hexadecimal)

| Header | ID | Data Length | Instruction | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0XFE | 0X04 | 0X03 | 0X05 0X01 | 0XF4 |

Since commands are sent using broadcast IDs, no data will be returned.

Note: The memory table EPROM has a protection lock switch, which needs to be turned off (set to 0) before modifying the ID; otherwise, the example ID number will not be saved when power is lost.

For detailed operations, please refer to the memory table or operation manual of the specific servo model.

Example:Control Servo ID1 to rotate to position 2048 at a speed of 1000 per second.

The first address of the target location in the control table is  0X2A , so starting from address  0X2A , six consecutive ByteDance data are written,

are position data  0X0800(2048)  ;

Time Data0X0000(0) ;

Speed data0X03E8(1000).

The ID of the sent command packet uses a non-broadcast ID (0xFE), so the servo will return a status packet after the command reception is completed.

Command Frame: FF FF 01 09 03 2A 00 08 00 00 E8 03 D5 (Sent in Hexadecimal)

| Header | ID | Data Length | Instruction | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X09 | 0X03 | 0X2A | 0XD5 |
|  |  |  |  | 0X00 0X08 |  |
|  |  |  |  | 0X00 0X00 |  |
|  |  |  |  | 0XE8 0X03 |  |

Returned Data Frame:FF FF 01 02 00 FC (Hexadecimal Display)

| Header | ID | Data Length | Status | Checksum |
| --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X02 | 0X00 | 0XFC |

Returning a working state of 0 indicates that the servo has correctly received the command without error and has begun execution.

### 2.2.4 Asynchronous Write Instruction REG WRITE

The REG WRITE instruction is similar to WRITE DATA, except that the execution time is different. When a REG WRITE instruction frame is received, the received data is stored in the buffer for later use, and the Registered Instruction register is set to 1. When an ACTION instruction is received, the stored instruction is finally executed.

Length    N+3 (N is the number of data to be written)

Command    0X04

Parameter 1 The starting address of the data write area

Parameter 2 The first data to be written

Parameter 3 The second data to be written

Parameter N+1 The Nth data to be written

Example: Control servos from ID1 to ID10 to rotate to position 2048 at a speed of 1000 per second. Only IDs on the bus in the following command packet will receive the command and return, while other IDs not on the bus will not return.

ID 1 Asynchronous Write Instruction Packet:  FF FF 01 09 04 2A 00 08 00 00 E8 03 D4

ID 1       Return Packet:FF FF 01 02 00 FC

ID 2 Asynchronous Write Instruction Packet:  FF FF 02 09 04 2A 00 08 00 00 E8 03 D3

ID 3 Asynchronous Write Instruction Packet:FF FF 03 09 04 2A 00 08 00 00 E8 03 D2

ID 4 Asynchronous Write Command Packet:FF FF 04 09 04 2A 00 08 00 00 E8 03 D1

ID 5 Asynchronous Write Instruction Packet:FF FF 05 09 04 2A 00 08 00 00 E8 03 D0

ID 6 Asynchronous Write Instruction Packet:FF FF 06 09 04 2A 00 08 00 00 E8 03 CF

ID 7 Asynchronous Write Instruction Packet:FF FF 07 09 04 2A 00 08 00 00 E8 03 CE

ID 8 Asynchronous Write Instruction Packet:FF FF 08 09 04 2A 00 08 00 00 E8 03 CD

ID 9 Asynchronous Write Instruction Packet:FF FF 09 09 04 2A 00 08 00 00 E8 03 CC

ID10 Asynchronous Write Instruction Packet:FF FF 0A 09 04 2A 00 08 00 00 E8 03 CB

### 2.2.5 Execute asynchronous write instruction ACTION

Function          Triggers REG WRITE instruction

Length 0X02

Instruction 0X05

Parameter  None

The ACTION command is very useful when controlling multiple servos simultaneously.

When controlling multiple servos, using the ACTION command allows the first and last servos to execute their respective actions simultaneously, with no delay in between.

When sending ACTION commands to multiple servos, the broadcast ID (0xFE) is used. Therefore, sending this command will not result in a data frame being returned.

Example: After sending the asynchronous write command to control servos from ID1 to ID10 to rotate to position 2048 at a speed of 1000 per second, when an asynchronous write command needs to be executed,

The following instruction packet (  FF FF FE 02 05 FA ) needs to be sent. All servos on the bus will execute the previously received asynchronous write instruction after receiving this instruction.

### 2.2.6 Synchronous Write Instruction SYNC WRITE

Function  is used to control multiple servos simultaneously.

ID      0XFE

Length (L + 1) * N + 4 (L: Data length sent to each servo, N: Number of servos)

Command 0X83

Parameter 1 The starting address for writing data

Parameter 2 Length (L) of the written data

Parameter 3 ID number of the first servo

Parameter 4  writes the first data to the first servo

Parameter 5  writes the second data of the first servo

...

Parameter L+3  writes the Lth data of the first servo

Parameter L+4 ID number of the second servo

Parameter L+5  writes the first data of the second servo

Parameter L+6  writes the second data of the second servo

…

Parameter 2L+4  Write to the L-th data of the second servo

…

Different from the REG WRITE+ACTION instruction, the SYNC WRITE instruction has higher real-time performance. A single SYNC WRITE instruction can modify the contents of the control tables of multiple servos at once, while the REG WRITE+ACTION instruction does this in steps. Nevertheless, when using the SYNC WRITE instruction, the length of the written data and the starting address of the saved data must be the same.

Example:  Write position 0X0800, time 0X0000, and speed 0X03E8 to the starting address 0X2A of a total of 4 servos from ID1 to ID4 ( low-order ByteDance first, high-order ByteDance last ).

Command Frame: FF FF FE 20 83 2A 06 01 00 08 00 00 E8 03 02 00 08 00 00 E8 03 03 00 08 00 00 E8 03 04 00 08 00 00 E8 03  58 (Sent in Hexadecimal)

| Header | ID | Data Length | Instruction | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0XFE | 0X20 | 0X83 | 0X2A 0X06 | 0X58 |
|  |  |  |  | 0X01 0X00 0X08 0X00 0X00 0XE8 0X03 |  |
|  |  |  |  | 0X02 0X00 0X08 0X00 0X00 0XE8 0X03 |  |
|  |  |  |  | 0X03 0X00 0X08 0X00 0X00 0XE8 0X03 |  |
|  |  |  |  | 0X04 0X00 0X08 0X00 0X00 0XE8 0X03 |  |

Since commands are sent using broadcast IDs, no data will be returned.

### 2.2.7 Synchronous Read Instruction SYNC READ

Function  is used to query multiple servos simultaneously.

ID      0XFE

Length    N + 4 (N: Number of Servos)

Instruction 0X82

Parameter 1  The first address of the read data

Parameter 2: Length of Read Data

Parameter 3  ID number of the first servo

Parameter 4 ID number of the second servo

…

Parameter N+2 ID number of the Nth servo

…

A SYNC READ command can query the control table contents of multiple servos at once. The SYNC READ command specifies the IDs of the servos to be queried, and the servos return response packets in the order of the IDs in the command packet. When using the SYNC READ command, the length and starting address of all queried data must be the same (this command is available for some serial bus servos).

Example: Query the first address 0X38 of a total of 2 servo motors from ID1 to ID2, and the current position, current speed, current load, current voltage, and current temperature, a total of 8 bytes of data (low-order bytes first, high-order bytes last).

Synchronous Read Instruction Packet:  FF FF FE 06 82 38 08 01 02 36

| Header | ID | Data Length | Instruction | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0XFE | 0X06 | 0X82 | 0X38 | 0X36 |
|  |  |  |  | 0X08 |  |
|  |  |  |  | 0X01 |  |
|  |  |  |  | 0X02 |  |

Return Packet:

FF FF 01 0A 00 00 08 00 00 00 00 79 1E 55

FF FF 02 0A 00 FF 07 00 00 00 00 77 23 53

Return packet disassembly: ID1 return packet:FF FF 01 0A 00 00 08 00 00 00 00 79 1E 55

| Header | ID | Data Length | Status | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X0A | 0X00 | 0X00 0X08 | 0X55 |
|  |  |  |  | 0X00 0X00 |  |
|  |  |  |  | 0X00 0X00 |  |
|  |  |  |  | 0X79 |  |
|  |  |  |  | 0X1E |  |

ID2返回包：FF FF 02 0A 00 FF 07 00 00 00 00 77 23 53

| Header | ID | Effective Data Length | Status | Parameter | Checksum |
| --- | --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X02 | 0X0A | 0X00 | 0XFF 0X07 | 0X53 |
|  |  |  |  | 0X00 0X00 |  |
|  |  |  |  | 0X00 0X00 |  |
|  |  |  |  | 0X77 |  |
|  |  |  |  | 0X23 |  |

### 2.2.8 Reset Instruction RESET

Function Reset specific data in the memory control table (used for specific servo models)

Length 0X02

Instruction 0X06

Parameter  None

Example:  Reset the servo with ID number 0.

Command Frame:FF FF 01 02 06 F6 (Sent in Hexadecimal)

| Header | ID | Data Length | Instruction | Checksum |
| --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X00 | 0X02 | 0X06 | 0XF7 |

Returned data frame:FF FF 01 02 00 FC (Hexadecimal display)

| Header | ID | Data Length | Status | Checksum |
| --- | --- | --- | --- | --- |
| 0XFF 0XFF | 0X01 | 0X02 | 0X00 | 0XFC |